# Configuration File Format
Multi-port Models

The following table shows the CONFIG file details for models with multiple parallel ports (no serial port). This information is provided for users who use FTP for configuration, and thus need to edit the CONFIG file directly.

Note: The CONFIG file will vary with the firmware version installed in your Print Server, so may not exactly match this description.

## Explanation

The information in each column is as follows:

Default Line

The line as obtained from the device. Each line has the following structure:
Line_number Token:Parameter

Where
Line_number is a system parameter and MUST NOT be changed.
Token is a readable identifier for the line.
Parameter is the current setting. A colon ( : ) separates the Token from the Parameter.

Allowable Values

This column lists the values which may be used for the Parameter. The following terms are used:

Text[n], where n is a number, indicates a text field with a maximum length of n characters.
Numeric[0..n], where n is a number, indicates the acceptable range of values for a numeric field.
Access Code, where the allowable values are: Read Only, Read/Write, Not Accessible.

Description

Explanation of the purpose of the parameter.

## Data

| Default Line | Allowable Values | Description |
| --- | --- | --- |
| 0001 BOX_NAME:SCxxxxxx | Text [19] | Device name |
| 0011 IPXSPX_P:Enable | Enable, Disable | NetWare protocol |
| 0012 TCPIP_P :Enable | Enable, Disable | TCP/IP protocol |
| 0013 APTALK_P:Enable | Enable, Disable | AppleTalk protocol |
| 0014 NETB_P:Enable | Enable, Disable | NetBEUI protocol |
| 0100 L1_PROUT:P1 | P1, P2, P3 (if exist) | Port used by Logical Printer 1 |
| 0101 L1_PREST: | Text [15] (hex) | Pre-string LP 1 |
| 0102 L1_POSTR: | Text [15] (hex) | Post-string LP 1 |
| 0103 L1_CHGLF:No | Yes, No | Convert LF to LF/CR |
| 0120 L2_PROUT:P1 | P1, P2, P3 (if exist) | Port used by Logical Printer 2 |
| 0121 L2_PREST: | Text [15] (hex) | Pre-string LP 2 |
| 0122 L2_POSTR: | Text [15] (hex) | Post-string LP 2 |
| 0123 L2_CHGLF:No | Yes, No | Convert LF to LF/CR |
| 0140 L3_PROUT:P1 | P1, P2, P3 (if exist) | Port used by Logical Printer 3 |
| 0141 L3_PREST: | Text [15] (hex) | Pre-string LP 3 |
| 0142 L3_POSTR: | Text [15] (hex) | Post-string LP 3 |
| 0143 L3_CHGLF:No | Yes, No | Convert LF to LF/CR |
| 0160 L4_PROUT:P1 | P1, P2, P3 (if exist) | Port used by Logical Printer 4 |
| 0161 L4_PREST: | Text [15] (hex) | Pre-string LP 4 |
| 0162 L4_POSTR: | Text [15] (hex) | Post-string LP 4 |
| 0163 L4_CHGLF:No | Yes, No | Convert LF to LF/CR |
| 0180 L5_PROUT:P1 | P1, P2, P3 (if exist) | Port used by Logical Printer 5 |
| 0181 L5_PREST: | Text [15] (hex) | Pre-string LP 5 |
| 0182 L5_POSTR: | Text [15] (hex) | Post-string LP 5 |
| 0183 L5_CHGLF:No | Yes, No | Convert LF to LF/CR |
| 0200 L6_PROUT:P1 | P1, P2, P3 (if exist) | Port used by Logical Printer 6 |
| 0201 L6_PREST: | Text [15] (hex) | Pre-string LP 6 |
| 0202 L6_POSTR: | Text [15] (hex) | Post-string LP 6 |
| 0203 L6_CHGLF:No | Yes, No | Convert LF to LF/CR |
| 0220 L7_PROUT:P1 | P1, P2, P3 (if exist) | Port used by Logical Printer 7 |
| 0221 L7_PREST: | Text [15] (hex) | Pre-string LP 7 |
| 0222 L7_POSTR: | Text [15] (hex) | Post-string LP 7 |
| 0223 L7_CHGLF:No | Yes, No | Convert LF to LF/CR |
| 0240 L8_PROUT:P1 | P1, P2, P3 (if exist) | Port used by Logical Printer 8 |
| 0241 L8_PREST: | Text [15] (hex) | Pre-string LP 8 |
| 0242 L8_POSTR: | Text [15] (hex) | Post-string LP 8 |
| 0243 L8_CHGLF:No | Yes, No | Convert LF to LF/CR |
| 2000 NOP_MODE:PS | PS, RP | NetWare mode Print Server (PS)or Remote Printer (RP) |
| 2001 NFREthII:Enable | Enable, Disable | Ethernet II frame type |
| 2002 NFR802.2:Enable | Enable, Disable | 802.2 frame type |
| 2003 NFR802.3:Enable | Enable, Disable | 802.3 frame type |
| 2004 NFRSNAP :Enable | Enable, Disable | SNAP frame type |
| 2101 NFS_NAME: | Text [20] | Master file server |
| 2102 N_NOTIFY:No | Yes, No | Notification by node address |
| 2103 N_FREQ :1 | Numeric [0..255] | Polling queue interval |
| 2110 NDS_TREE: | Text [39] | NDS Tree Name |
| 2111 NCONTEXT: | Text [235] | NDS context |
| 2501 NR_NAME1: | Text [19] | NetWare Print Server for parallel port 1 |
| 2502 NR_NAME1: | Text [19] | NetWare Print Server for parallel port 2 |
| 2503 NR_NAME1: | Text [19] | NetWare Print Server for serial port |
| 2504 NR_NAME1: | Text [19] | NetWare Print Server for parallel port 3 |
| 3000 AP_ZONE:* | Text [19] | AppleTalk zone |
| 3001 AP_TYPE1: LaserWriter | Text [19] | Printer type for parallel port 1 |
| 3002 AP_TYPE2: LaserWriter | Text [19] | Printer type for parallel port 2 |
| 3004 AP_TYPE4 LaserWriter | Text [19] | Printer type for parallel port 3 |
| 3101 AP_PCOMM1:No | Yes, No | ASCII (No) or Binary (Yes) communication for parallel port 1 |
| 3102 AP_PCOMM2:No | Yes, No | ASCII (No) or Binary (Yes) communication for parallel port 2 |
| 3104 AP_PCOMM4:No | Yes, No | ASCII (No) or Binary (Yes) communication for parallel port 3 |
| 4000 IP_ADDR:0.0.0.0 | IP Address | Device IP Address |
| 4001 GATEWAY:0.0.0.0 | IP Address | Gateway |
| 4002 MASK :0.0.0.0 | IP Mask | Subnet mask |
| 4010 TCP_INT:2 | Numeric [0..255] | Delay before reconnection attempt |
| 4011 TCP_CNT:254 | Numeric [0..255] | No. of reconnection attempts. |
| 4100 MAIL_IP:0.0.0.0 | IP Address | Mail A/C IP Address |
| 4101 MAIL_ACC: | Text [19] | Mail A/C Name |
| 4102 MAIL_PAS:******** | Text [19] | Mail A/C Password |
| 4103 MAIL_INT:0 | Numeric [0..64K] | Check Mail Interval |
| 4104 MAIL_BAN:No | Yes, No | Mail Banner Page |
| 4105 MAIL_RED: | Text [19] | Mail A/C name for redirection |
| 4106 MAIL_POR:P1 | P1..P3 (if exist) L1..L8 (if exist) | Printer port (physical or logical) for Internet (Mail) Printing |
| 4107 MAIL_EVR:No | Yes, No | Print every E Mail |
| 4108 MAIL_NOT:No | Yes, No | Notify mail printed |
| 4109 MAIL_MOD: | Text [19] | Mail Printer model |
| 5000 SMBGNAME: | Text [19] | NetBEUI group (domain) name |
| 5001 SMBDROP:No | Yes, No | Abort print job if error |
| 5002 SMBDELAY:0 | Numeric [0..255] | Delay time |
| 6000 CONTACT: | Text [31] | SNMP person |
| 6001 LOCATION: | Text [31] | Contact location |
| 6011 M1_IP:0.0.0.0 | IP Address | Management station 1 |
| 6012 M1_ACCP: Not Accessible | Access Code | Access rights |
| 6013 M1_CSTR: | Text [7] | Community String |
| 6021 M2_IP:0.0.0.0 | IP Address | Management station 2 |
| 6022 M2_ACCP: Not Accessible | Access Code | Access rights |
| 6023 M2_CSTR: | Text [7] | Community String |
| 6031 M3_IP:0.0.0.0 | IP Address | Management station 3 |
| 6032 M3_ACCP: Not Accessible | Access Code | Access rights |
| 6033 M3_CSTR: | Text [7] | Community String |
| 6041 M4_IP:0.0.0.0 | IP Address | Management station 4 |
| 6042 M4_ACCP: Not Accessible | Access Code | Access rights |
| 6043 M4_CSTR: | Text [7] | Community String |
| 6111 T1_IP:0.0.0.0 | IP Address | Trap 1 Address |
| 6112 T1_S:0 | Numeric[0..2] | Severity Level |
| 6113 T1_CSTR: | Text [7] | Community String |
| 6114 T1_ENAB:Disable | Enable, Disable | Trap receive station 1 |
| 6121 T2_IP:0.0.0.0 | IP Address | Trap 2 Address |
| 6122 T2_S:0 | Numeric[0.2] | Severity Level |
| 6123 T2_CSTR: | Text [7] | Community String |
| 6124 T2_ENAB:Disable | Enable, Disable | Trap receive station 2 |
| 6131 T3_IP:0.0.0.0 | IP Address | Trap 3 Address |
| 6132 T3_S:0 | Numeric[0.2] | Severity Level |
| 6133 T3_CSTR: | Text [7] | Community String |
| 6134 T3_ENAB:Disable | Enable, Disable | Trap receive station 3 |
| 6141 T4_IP:0.0.0.0 | IP Address | Trap 4 Address |
| 6142 T4_S:0 | Numeric[0.2] | Severity Level |
| 6143 T4_CSTR: | Text [7] | Community String |
| 6144 T4_ENAB:Disable | Enable, Disable | Trap receive station 4 |