# Unix - TCP/IP

This document explains how to configure and use the PrintMate in the Unix TCP/IP environment.

## Software Requirements

- TCP/IP protocol, FTP (Optional: - BOOTP, Rarp, Telnet)

- LPD printing system (for LPD printing)

- C Compiler (for PSfilter proprietary printing system only).

## Device Configuration

Configuration of your PrintMate can be done by any of the following methods. Follow each link for further details.

| BiAdmin | If you have Windows 95/98/NT, this is the recommended program for configuration and management. |
| --- | --- |
| FTP | Command-line FTP (File Transfer Protocol) clients are supported. FTP can be used to assign an IP Address to the PrintMate, provided that the Unix Host Configuration (described later in this document) has been completed. |
| Web Browser | The PrintMate incorporates a HTTP server, allowing you to configure it with a Web Browser. Most browser will work, provided they support tables and forms. This method can be used to assign an IP Address to the PrintMate, provided that the Unix Host Configuration (described later in this document) has been completed. |

### IP Address Configuration

- The PrintMate allows the following IP Addresses to be entered. (The entry in brackets shows the line number and token in the device’s configuration file. This information is for people using FTP.)

| - IP Address (4000 IP_ADDR) | - The PrintMate is able to obtain an IP Address dynamically, or use a static IP Address assigned during configuration. Because of the support for dynamic IP Addresses, the PrintMate ships with an IP Address of 0.0.0.0. - See the following section for more details on using either a static or dynamic IP Address. |
| --- | --- |
| - Gateway Address (4001 GATEWAY) | - If your network segment has a router, enter the router address here. If there is no router, leave the address as 0.0.0.0. |
| - Subnet Mask (4002 MASK) | - If the Gateway Address above is 0.0.0.0 the Subnet Mask should also be left at 0.0.0.0. If you have a router, enter the Subnet mask for the segment to which the PrintMate is attached. |

Device IP Address

- Normally, the PrintMate is assigned a static IP Address. If you leave its IP Address at the default value of 0.0.0.0, its sequence for finding an IP Address dynamically is as follows:

DHCP,  BOOTP,  RARP.

See Dynamic IP Address Configuration on page 4 for more details.

### Other Configuration Settings

When assigning the IP Addresses, you should also check the following configuration settings.

| Device Name (0001 BOX_NAME) | The default name (Server Name) consists of 8 characters (letters and/or numbers).. This can be changed. The new name MUST NOT exceed 19 characters, nor include any spaces. |
| --- | --- |
| TCP Session Retry Interval (4010 TCP_INT) | Sets how long the PrintMate should wait before retrying a TCP/IP connection which is lost. Allowable values are from 0 to 255 seconds, with 2 as the default. |
| TCP Session Retry Count (4011 TCP_CNT) | Sets how many attempts at reconnection will be made. After that, the TCP/IP session will be terminated. Allowable values are from 0 to 255, with 254 as the default. |

Serial Port Setup (if exists)

| Baud Rate (bps) (0030 BAUDRATE) | Depending on the model, allowable values will range from 300 to 38400, or 1200 to 460800. |
| --- | --- |
| Stop bits (0031 STOPBITS) | Allowable values are 1 or 2 |
| Parity (0032 PARITY) | Allowable values are NONE, ODD, EVEN |
| Data bits (0033 DATABITS) | Allowable values are 7 or 8 |
| Handshake (0034 HANDSHAK) | Allowable values are NONE, HARDWARE, XON/XOFF, BOTH |

### Logical Printers

Logical Printers can be used to create a “Virtual” printer. For example, to create a Landscape printer, you could define a Logical Printer as follows:

| Pre-string | Printer Control codes to switch the printer to Landscape mode |
| --- | --- |
| Post-string | Printer Control codes to reset the printer, restoring the default settings. |

Another logical printer could be used to print Unix-format text files for a DOS printer, by converting Unix-style LF (Line Feeds) to DOS-style LF/CR (Line Feed, Carriage Return) pairs.

PrintMate models with 1 parallel port support 3 Logical Printers, otherwise they support 8 Logical Printers. The names (L1..L8) cannot be changed. Each Logical Printer has 4 settings as shown below.

Logical Printer Setup Data

| Physical Port (0100 L1_PROUT) | PrintMate Port to which the printer is attached (P1 to P3 for parallel ports and SP for the serial port). |
| --- | --- |
| String Before Job (0101 L1_PREST) | The printer control string (in hex) to be sent to the printer before each print job. |
| String After Job (0102 L1_POSTR) | The printer control string (in hex) to be sent to the printer after each print job. |
| Convert LF to CR+LF (0103 L1_CHGLF) | If ON, LF (line feed) characters are changed to CR+LF (carriage return + line feed). If OFF, no conversion is done. |

|  | The maximum size of a printer control string is 15 characters. |
| --- | --- |

The line numbers in the PrintMate’s CONFIG file are listed below. This information is useful if you use FTP for configuration.

| Logical Printer | Line Numbers |
| --- | --- |
| L1 | 0100 to 0103 |
| L2 | 0120 to 0123 |
| L3 | 0140 to 0143 |
| L4 | 0160 to 0163 |
| L5 | 0180 to 0183 |
| L6 | 0200 to 0203 |
| L7 | 0220 to 0223 |
| L8 | 0240 to 0243 |

## Unix Host Configuration

The PrintMate must be provided with a IP Address so that it will be a valid device on the LAN. This can be done using either Static or Dynamic IP Address configuration.

### Static IP Address Configuration

- Examine the base of the PrintMate to find the Default Server Name and Hardware address. These are shown on a sticker on the base of the unit.

- Login to the UNIX host as root.

- Add the PrintMate to the /etc/hosts file by adding the following line to the file:

IP_Address NAME # comment

Where:
IP_Address is the IP address for the PrintMate. This address must match the address stored in the device.
NAME is the PrintMate’s name. This must match the name stored in the device.
 # comment. Add the Default Name as a comment.

Example:

192.10.2.54 PS_Rm203 #Default name PS123456

In the example above, PS123456 is assigned the IP address 192.10.2.54 and the name “PS_Rm203”.

- Associate the hardware address with the IP address of the PrintMate, by using the arp command as follows:

arp -s NAME 00:c0:02:xx:yy:zz

Where:
NAME is the name assigned to the device
00:c0:02:xx:yy:zz is the hardware address of the PrintMate, as shown on the sticker on the base of the device.

Example:

arp -s PS_Rm203 00:c0:02:12:34:56

- Check the IP Address using the ping command:

ping NAME

You should receive a response. If you get a Timeout message, the above procedure has failed.

- If the PrintMate is not configured yet, you can now connect and configure it using FTP. For 100BaseT models, you can also use your Web Browser.

### Dynamic IP Address Configuration

If the device’s IP Address is left at 0.0.0.0 it will try to obtain a dynamic IP Address by using the following methods in sequence: DHCP, BOOTP, RARP.

Using DHCP

Using DHCP is only possible if you have DHCP management software which allows you to take advantage of this feature. Otherwise, the PrintMate’s IP Address will be unknown, and connection to it will be impossible. In this case, configure the PrintMate for a static IP Address. (See the preceding section.)

Using BOOTP

- Perform steps 1 to 3 for assigning a static IP Address.

- Add the following entry to the Boot Table  /etc/bootptab

NAME:ht=ether:vm=rfc1024::ha=PA:ip=IP:sm=SM:gw=GW

Where
NAME is the PrintMate’s name
PA is the hardware address of the PrintMate
IP is the PrintMate’s IP Address
SM is the Subnet Mask
GW is the Gateway IP Address

- If it is not running, start the bootp daemon (the usual command is bootpd) and then reset the PrintMate. It should then acquire an IP Address using bootp.

- If the PrintMate is not configured yet, you can now configure it using FTP. or your Web Browser. Otherwise, check with the ping command:

ping NAME

You should receive a response. If you get a Timeout message, the above procedure has failed.

Using RARP

- Perform steps 1 to 3 for assigning a static IP Address.

- If the rarp daemon is not running, start it with the command:

rarpd -a

- Add a line to the Ethernet Address table /etc/ethers

00:c0:02:xx:yy:zz NAME

Where
00:c0:02:xx:yy:zz is the hardware address of the PrintMate
NAME is the name of the PrintMate.

- Reset the PrintMate. When it reboots, it should acquire an IP Address from rarp.

- If the PrintMate is not configured yet, you can now configure it using FTP or your Web Browser. Otherwise, check the Address with the ping command:

ping NAME

You should receive a response. If you get a Timeout message, the above procedure has failed.

## Printing Methods – Overview

There are 4 printing methods to choose from. All are explained in the following sections. Select whichever is the most convenient in your environment.

LPD

LPD is a standard print method for most UNIX systems. The benefit of this method is that it eliminates the need to install additional software on the host.
The problem is that in most cases the LPD protocol sends out the data file before the control file. The PrintMate will print the data file immediately, ignoring any print options set in the control file. To solve this problem, use PSfilter instead.

Direct Socket Interface

The Direct Socket Interface (DSI) is a Unix-based method of providing a “direct” connection between a host computer and a printer. The host and the PrintMate establish a TCP connection, using a special socket number. All data sent over this connection is treated as print data, and sent transparently to a logical printer defined on the PrintMate.

PSfilter

PSfilter is a proprietary print method provided with the PrintMate. The benefit of this method is that it provides many print options such as banner print, copies, and so on. To use PSfilter, a C compiler must be installed on every UNIX host so that the PSfilter source code can be compiled into executable code and installed on the UNIX host.

FTP (File Transfer Protocol)

FTP is also a standard print method in most UNIX systems, but it is NOT recommended except as a test and back-up method of printing.

LPD and PSfilter work well with a large number of users because they both employ print queue processes. FTP does not implement a print queue. If the printer is busy, the print command will fail, creating an error situation.

See the FTP document for details on using FTP.

## LPD Printing Configuration

LPD is a built-in printing protocol for most UNIX systems, and is also supported in Windows NT 3.5 or later.

### LPD on IBM AIX 4.15

Before proceeding, ensure that the PrintMate has been assigned an IP Address. To setup your AIX system for LPD printing, perform the following steps.

- Add the PrintMate to /etc/hosts.lpd, using the name you assigned to the PrintMate.

- Start the LPD daemon if it is not running, using the following command:

start src -s qdaemon

- Start the system administration tool smit and select Print Spooling

- Create the required number of queues (one for each logical printer) by selecting:

- Add a Print Queue

- Remote (Printer attached to Remote Host)

- Standard Processing

- Use the following information:

| Field | Entry |
| --- | --- |
| Name of queue to add | Use a single-word queue name which indicates which printer is attached. |
| Hostname for remote server | PrintMate name as used in /etc/hosts.lpd. |
| Name of queue on remote server | Logical printer number (L1..L3 or L1..L8) to service this queue. |
| Type of print spooler on remote server. | Use default value (AIX Version xxx) |

- Ensure that the logical printers are configured in the PrintMate. Refer to Logical Printers earlier in this document.

- Print using the following command:

lp -d printer_queue file_name

Where
printer_queue is one of the entries used in Name of queue to add.
file_name is the file you wish to print.

### LPD on System V

Before beginning LPD Setup, ensure that an IP Address has been assigned to the PrintMate. Keep the following points in mind:

- The remote host name is the name of the PrintMate.

- The remote printer name is the print queue name for the Logical Printer. Logical printers also need to be configured on the PrintMate itself. (See page 3).

- If your UNIX asks for the LPD type, be sure to identify the service type as BSD. The PrintMate’s LPD protocol meets BSD system standards.

- In the sample commands shown, printer_name is the name of the Print Queue serviced by the PrintMate, and Spooler_directory is the name of the directory used to spool the print jobs.

Procedure

| Action | Sample Command |
| --- | --- |
| Stop Print Services | /usr/lib/lpshut |
| Add a System Printer | /usr/lib/lpadmin -p printer_name -v /dev/null |
| Restart the Print Services | /usr/lib/lpsched |
| Enable printing to the new printer device | enable printer_name |
| Start accepting jobs for the new printer device | accept printer_name |
| Create a spooling directory | mkdir /usr/spool/Spooler_directory |
| Make spooling daemon the owner of this directory | chown daemon /usr/spool/Spooler_directory |
| Create read/write permissions | chmod 775 /usr/spool/Spooler_directory |
| Give permissions to LPD processes. | chgrp daemon /usr/spool/Spooler_directory |
| Add remote printer(s) | See following section. |

Adding Remote Printers

A remote printer is added by inserting the following line in the /etc/printcap file.

|  | The entry is really one line, but can be entered as shown. Use a TAB character where shown. |
| --- | --- |

printer_name¦Remote_Printer_Alias:\ [Tab] :lp=:\ [Tab] :rm=PS_NAME:\ [Tab] :rp=Logical_Printer_name:\ [Tab] :sd=Spooler_directory:\ [Tab] :mx#0:

Where:

printer_name is the Print Queue name used to store jobs for the corresponding logical printer
PS_NAME is the PrintMate name defined in /etc/hosts
Logical_Printer_name is the logical printer name on the PrintMate (L1..L3 or L1..L8, depending on your model)
Spooler_directory is the directory you created in Step 6.

Example:

Marketing¦RP1_PS123456:\ [Tab] :lp=:\ [Tab] :rm=PS_Rm203:\ [Tab] :rp=L1:\ [Tab] :sd=/usr/spool/Marketing:\ [Tab] :mx#0:

Repeat this process for each Logical Printer/Print Queue combination that you wish to create.

### LPD on BSD

Before continuing, ensure that an IP Address has been assigned to the PrintMate. Remember the following:

- The remote host name is the name of the PrintMate.

- The remote printer name is the logical printer (L1..L3, or L1..L8) on the PrintMate.

- If asked for the LPD type, enter the service type as BSD.

- In the sample commands shown, printer_name is the Print Queue serviced by the logical printer on the PrintMate, and Spooler_dir is the name of the directory used to spool the print jobs.

Procedure

| Action | Sample Command |
| --- | --- |
| Create a spooling directory | mkdir /usr/spool/Spooler_dir |
| Set spooling daemon as owner of this directory. | chown daemon /usr/spool/Spooler_dir |
| Create read/write permissions. | chmod 775 /usr/spool/Spooler_dir |
| Give permissions to LPD processes. | chgrp daemon /usr/spool/Spooler_dir |
| Add remote printer(s) | See below. |
| Start lpc print mechanism | lpc start printer_name |

Adding Remote Printers

A remote printer is added by inserting the following line in the /etc/printcap file.

|  | The entry is really one line, but can be entered as shown. Use a TAB character where shown. |
| --- | --- |

printer_name¦Remote_Printer_Alias:\ [Tab] :lp=:\ [Tab] :rm=PS_NAME:\ [Tab] :rp=Logical_Printer_name:\ [Tab] :sd=Spooler_directory:\ [Tab] :mx#0:

Where:

printer_name is the Print Queue name used to store jobs for the corresponding logical printer
PS_NAME is the PrintMate name defined in /etc/hosts
Logical_Printer_name is the logical printer name on the PrintMate (L1..L3 or L1..L8, depending on your model)
Spooler_directory is the directory you created in Step 6.

Example:

Marketing¦RP1_PS123456:\ [Tab] :lp=:\ [Tab] :rm=PS_Rm203:\ [Tab] :rp=L1:\ [Tab] :sd=/usr/spool/Marketing:\ [Tab] :mx#0:

Repeat this process for each Logical Printer/Print Queue combination that you wish to create.

### Printing using LPD

For LPD printing instructions, refer to your UNIX manual. The following example is for a BSD system:

lpr  -P printer_name filename

Where:
printer_name is the name of the Print Queue defined on the Unix host.
filename is the name of the file you wish to print.

Example:

lpr  -P Marketing  /etc/hosts

In the above example, the /etc/hosts file is sent to the printer queue Marketing. It will then be sent to the logical printer associated with this queue.

## Other Printing Methods

The PrintMate supports 2 other printing methods:

- DSI (Direct Socket Interface)

- PSfilter (Proprietary print method)

### Printing Using DSI

Logical printers must be configured on the PrintMate as explained in Logical Printers earlier in this document. Your model may support 8 logical Printers. However, DSI on the PrintMate supports only 3 Logical Printers

Socket numbers are defined as follows:

| Logical Printer No. | Socket No. |
| --- | --- |
| 1 | 4010 |
| 2 | 4020 |
| 3 | 4030 |

### Printing Using PSfilter

PSfilter is a proprietary printing system which provides many print options. To use PSfilter, a C compiler and Socket Library must exist on the host so that psfilter.c can be compiled to an executable file.

The PSfilter program is shipped in source code as psfilter.c. Detailed instructions for installing, configuring, and using PSfilter are provided in the following files. These files, along with PSfilter.c, are stored on the CD-ROM in the following directories:

\driver\lpti\lpsource   (text format)

\driver\lpti\tar   (tar format)

| Operating System | Filename |
| --- | --- |
| SCO UNIX System V Release 3 | SCO.TXT |
| HP UXIX on HP workstation | HP.TXT |
| Sun 5.x on Sun SPARC workstation | SUN5.TXT |
| Sun 4.x on Sun SPARC workstation | SUN4.TXT |
| AT&T UNIX SV Release 4 | SVR4.TXT |
| DEC/OSF1 | DECOSF1.TXT |
| IBM AIX | AIX.TXT |
| UNIXWare | UNIXWARE.TXT |

|  | If your system is not listed above, do not attempt to use the PSfilter method of printing. |
| --- | --- |

## Telnet

Telnet can be used to monitor the status of the printers attached to the PrintMate.

### Operation

Establish a connection to the PrintMate, by starting your Telnet program and providing the IP Address of the PrintMate. (No port number is required.)

e.g.

telnet 192.168.0.21

The PrintMate will respond with "Welcome to Print Server".

From the resulting prompt, 3 commands are possible:

- Help: Show brief help

- Monitor: show printer status

- Exit: leave telnet

Monitoring

The "Monitor" command will show the status of the printer, on each port, as shown in the following example.

(P1)STATE: Idle TYPE: Parallel PRINTER STATUS: Out Of Paper (P2)STATE: Printing TYPE: Parallel PRINTER STATUS: On-Line BYTES SERVICED: 75264 OCCUPIED BY: LPD (SP)STATE: Idle TYPE: Serial PRINTER STATUS: Offline

The display is updated once per second.